PRL Aluminum

Master specification

SECTION 08 32 13 - SLIDING ALUMINUM-FRAMED GLASS DOORS

GENERAL

SUMMARY

Section Includes:

Exterior rated, factory glazed multi panel aluminum framed sliding glass doors with top  rolling hardware.

Related sections:

Section 07 62 00 metal flashings and trim

Section 07 92 00 joint sealants

Section 08 11 16 aluminum doors

Section 08 43 13 glazed aluminum storefronts

Section 08 44 13 glazed aluminum curtain walls

Section 08 51 13 aluminum windows

Section 08 80 50 glass and glazing

REFERENCE STANDARDS

Aluminum Association (AA)

DAF 45 designated system for aluminum finishes

American Architectural Manufactures Association (AAMA)

AAMA/WDMA/CSA-101/I.S.2/A440-05 standard/specification for windows, doors and unit skylights

AAMA-501-05 Methods of test for exterior walls

AAMA-502-02 Voluntary specification for field testing of windows and sliding glass doors.

AAMA TIR A-11-04 maximum allowable deflection of framing systems for building cladding components at design wind loads

AAMA 2603 voluntary specifications, performance requirements and test procedures for pigmented organic coatings on aluminum extrusions and panels

AAMA 2604 voluntary specifications, performance requirements and test procedures for pigmented organic coatings on aluminum extrusions and panels

AAMA 2605 voluntary specifications, performance requirements and test procedures for pigmented organic coatings on aluminum extrusions and panels

AAMA CW-10 care and handling of architectural aluminum from shop to site

National Fenestration Rating Council (NFRC)

NFRC 100 procedure for determining fenestration product U-factors

NFRC 200 procedure for determining fenestration product solar heat gain (SHGC) and visible transmittance (VT) at normal incidence.

American Society for Testing and Materials (ASTM)

ASTM B 221-08 specification for aluminum-alloy extruded bars, rods, wire, profiles and tubes

ASTM E 283 standard test method for determining rate of air leakage through exterior windows, curtain walls, and doors under specified pressure specified pressure differences across the specimen.

ASTM E 330 standard test method for structural performance of exterior windows, doors, skylights and curtain walls by uniform static air pressure difference

ASTM E 331 standard test method for water penetration of exterior windows, skylights, doors, and curtain walls by uniform static air pressure difference

PERFORMANCE REQUIREMENTS

General Performance:  Aluminum-framed systems shall withstand the effects of the following performance requirements without exceeding performance criteria or failure due to defective manufacture, fabrication, installation, or other defects in construction:

Movements of supporting structure indicated on Drawings including, but not limited to, story drift and deflection from uniformly distributed and concentrated live loads.

Dimensional tolerances of building frame and other adjacent construction.

Failure includes the following:

Deflection exceeding specified limits.

Thermal stresses transferring to building structure.

Framing members transferring stresses, including those caused by thermal and structural movements to glazing.

Noise or vibration created by wind and by thermal and structural movements.

Loosening or weakening of fasteners, attachments, and other components.

Failure of operating units.

Delegated Design:  Design aluminum-framed systems, including comprehensive engineering analysis by a qualified professional engineer, using performance requirements and design criteria indicated.

Wind Loads:  70 M.P.H. exposure B, importance factor 1.

Deflection of Framing Members:

Deflection normal to the wall plane: limit to edge of glass in a direction perpendicular to glass plane shall not exceed L/175 of clear spans for spans up to 13 feet 6 inches or L/240 plus ¼" for spans over 13 feet six inches, or an amount that restricts glass edge deflection of individual lites to 3/4", whichever is less.

Deflection Parallel to Glazing Plane:  Limited to L/360 of clear span or 1/8 inch (3.2 mm), whichever is smaller, amount not exceeding that which reduces glazing bite to less than 75 percent of design dimension and that which reduces edge clearance between framing members and glazing or other fixed components directly below them to less than 1/8 inch (3.2 mm) and clearance between members and operable units directly below them to less than 1/16 inch (1.5 mm).

Structural-Test Performance:  Provide aluminum-framed systems tested according to ASTM E 330 as follows:

When tested to design pressure comply with 1.3.D.1

When tested at 150 percent of positive and negative wind-load design pressures, systems, including anchorage, do not evidence material failures, structural distress, and permanent deformation of main framing members exceeding 0.2 percent of span.

Test Durations:  10 seconds.

Doors shall be rated to AAMA/WDMA/CSA-101/I.S.2/A440-05 to the following:

R-15 when sill height is no greater than 0.75"

CW-35 when sill height is no greater than 1.56"

CW-45 when sill height is no greater than 1.86"

Air Infiltration:  Provide aluminum-framed systems with maximum air leakage through fixed glazing and operating framing areas of 0.30 cfm/sq. ft. (1.50 L/s per sq. m) of sample area when tested according to ASTM E 283 at a minimum static-air-pressure difference of 1.57 lbf/sq. ft. (75 Pa).

Water Penetration under Static Pressure:  Provide aluminum-framed systems that do not evidence water penetration through fixed glazing and operating framing areas when tested according to ASTM E 331 at a minimum static-air-pressure difference of the minimum required as specified in 1.3.F

SUBMITTALS

Product Data:  For each type of product indicated.

Qualification data: for installer

LEED Submittal:

Product Data for EA, MR, and IEQ Credits.

Shop Drawings:  For aluminum-framed systems.  Include plans, elevations, sections, details, and attachments to other work.

Include details of provisions for system expansion and contraction and for drainage of moisture in the system to the exterior.

Samples:  For each type of exposed finish required.

12 inches long for extrusions

12 inches x 12 inches for sheet products

2 inch x 4 inch samples for finishes

Other Action Submittals:

Entrance Door Hardware Schedule:  Prepared by or under the supervision of supplier, detailing fabrication and assembly of entrance door hardware, as well as procedures and diagrams.

Delegated-Design Submittal:  For aluminum-framed systems indicated to comply with performance requirements and design criteria, including analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

Product test reports.

Sample of field quality-control report format.

Maintenance data.

Warranties:  Sample of special warranties.

Evidence of ability to provide products that meet the energy requirements of the ENV-1/COMcheck form.

QUALITY ASSURANCE

Installer Qualifications:  Installers detailed information that shows competency to perform the work of this section

List projects completed that have same or similar products used on this project.

Testing Agency Qualifications:  Qualified according to ASTM E 699 for testing indicated.

Engineering Responsibility:  Prepare data for aluminum-framed systems, including Shop Drawings, based on testing and engineering analysis of manufacturer's units in systems similar to those indicated for this Project.

Product Options:  Information on Drawings and in Specifications establishes requirements for systems' aesthetic effects and performance characteristics.  Aesthetic effects are indicated by dimensions, arrangements, alignment, and profiles of components and assemblies as they relate to sightlines, to one another, and to adjoining construction.  Performance characteristics are indicated by criteria subject to verification by one or more methods including preconstruction testing, field testing, and in-service performance.

Source Limitations for Aluminum-Framed Systems:  Obtain from single source from single manufacturer.

Pre-installation Conference:  attend a pre-installation conference at a site to be determined.

WARRANTY

Special Warranty:  Installation contractors warranty in which installation contractor agrees to repair or replace components of aluminum-framed systems that do not comply with requirements or that fail in materials or workmanship within specified warranty period. The installation contractor may supplement his warranty with a pass through warrantee of the systems manufacturer.

Warranty Period:  One year from date of Substantial Completion.

Special Finish Warranty:  Installation contractors warrantee in which installation contractor agrees to repair or replace components on which finishes do not comply with requirements or that fail in materials or workmanship within specified warranty period.  Warranty does not include normal weathering. The installation contractor may supplement his warranty with a pass through warrantee of the systems manufacturer.

Warranty Period:  Five years from date of Substantial Completion.

PRODUCTS

MANUFACTURERS

Manufacturers:  Subject to compliance with requirements, provide products by one of the following

Basis-of-Design Product:  Subject to compliance with requirements, is the PRL Aluminum MAX slider CABO series or comparable product by one of the following:

Kawneer

Traco

Graham Architectural products.

Wausau Window and Wall systems

Requests for substitution of other manufactures must be made 15 days prior to bid date. Substitution requests must include:

test reports showing compliance with performance section

½ scale drawings showing profiles are comparable with basis of design product

Comparative analysis showing the performance characteristics Vs basis of design

Physical samples of proposed substitution.

MATERIALS

Aluminum:  Alloy and temper recommended by manufacturer for type of use and finish indicated.

Sheet and Plate:  ASTM B 209 (ASTM B 209M).

Extruded Bars, Rods, Profiles, and Tubes:  ASTM B 221 (ASTM B 221M).

Extruded Structural Pipe and Tubes:  ASTM B 429.

Structural Profiles:  ASTM B 308/B 308M.

Welding Rods and Bare Electrodes:  AWS A5.10/A5.10M.

Steel Reinforcement:  Manufacturer's standard zinc-rich, corrosion-resistant primer, complying with SSPC-PS Guide No. 12.00; applied immediately after surface preparation and pretreatment.  Select surface preparation methods according to recommendations in SSPC-SP COM and prepare surfaces according to applicable SSPC standard.

Structural Shapes, Plates, and Bars:  ASTM A 36/A 36M.

Cold-Rolled Sheet and Strip:  ASTM A 1008/A 1008M.

Hot-Rolled Sheet and Strip:  ASTM A 1011/A 1011M.

FRAMING SYSTEMS

Framing Members:  Manufacturer's standard extruded-aluminum framing members of thickness required and reinforced as required to support imposed loads.

Construction:  Non-thermal.

Glazing System:  Retained mechanically with gaskets on four sides.

Glazing Plane:  Center.

Hardware:

Doors run on 2 stainless steel adjustable tandem rollers.

Rollers are 2" diameter up to leaf weight of 499 pounds

Rollers are 3" diameter for leaf weights over 500 pounds

Lock devices is an MS-1850 stainless steel hook lock with thumb turn inside. Outside key lock is optional.

Pull handles are 1/2" dia offset wire pulls finished to match the primary extrusions.

- Note to specification writer: there are considerably more hardware options than the one above. Please contact PRL at 877-775-2586 for details, prices and options.

Brackets and Reinforcements:  Manufacturer's standard high-strength aluminum with non-staining, nonferrous shims for aligning system components.

Fasteners and Accessories:  Manufacturer's standard corrosion-resistant, non-staining, non-bleeding fasteners and accessories compatible with adjacent materials.

Use self-locking devices where fasteners are subject to loosening or turning out from thermal and structural movements, wind loads, or vibration.

Reinforce members as required to receive fastener threads.

Use exposed fasteners with countersunk Phillips screw heads fabricated from stainless steel.

Concrete and Masonry Inserts:  Hot-dip galvanized cast-iron, malleable-iron, or steel inserts, complying with ASTM A 123/A 123M or ASTM A 153/A 153M.

Concealed Flashing:  Manufacturer's standard corrosion-resistant, non-staining, non-bleeding flashing compatible with adjacent materials or Dead-soft, 0.018-inch- (0.457-mm-) thick stainless steel, ASTM A 240/A 240M of type recommended by manufacturer.

Framing System Gaskets and Sealants:  Manufacturer's standard, recommended by manufacturer for joint type.

Provide sealants for use inside of the weatherproofing system that have a VOC content of 250 g/L or less when calculated according to 40 CFR 59, Subpart D (EPA Method 24).

GLAZING SYSTEMS

Glazing:  As specified in Division 8 Section "Glazing."

Glazing Gaskets:  Manufacturer's standard marine type wrap around; replaceable, extruded, of profile and hardness required to maintain watertight seal.

ACCESSORY MATERIALS

Bituminous Paint:  Cold-applied, asphalt-mastic paint complying with SSPC-Paint 12 requirements except containing no asbestos; formulated for 30-mil (0.762-mm) thickness per coat.

FABRICATION

Form or extrude aluminum shapes before finishing.

Weld in concealed locations to greatest extent possible to minimize distortion or discoloration of finish.  Remove weld spatter and welding oxides from exposed surfaces by de-scaling or grinding.

Framing Members, General:  Fabricate components that, when assembled, have the following characteristics:

Profiles that are sharp, straight, and free of defects or deformations.

Accurately fitted joints with ends coped or mitered.

Means to drain water passing joints, condensation within framing members, and moisture migrating within the system to exterior.

Physical and thermal isolation of glazing from framing members.

Accommodations for thermal and mechanical movements of glazing and framing to maintain required glazing edge clearances.

Provisions for field replacement of glazing from exterior.

Fasteners, anchors, and connection devices that are concealed from view to greatest extent possible.

Mechanically Glazed Framing Members:  Fabricate for flush glazing without projecting stops.

Entrance Door Frames:  Reinforce as required to support loads imposed by door operation and for installing entrance door hardware.

ALUMINUM FINISHES

General: Comply with NAAMM's "metal finishes manual for architectural and metal products" for recommendations.

Bronze Anodic Finish:  AAMA 611, AA-M12C22A41, Class I, 0.018 mm or thicker.

Color: Dark bronze

EXECUTION

INSTALLATION

General:

Comply with manufacturer's written instructions.

Do not install damaged components.

Fit joints to produce hairline joints free of burrs and distortion.

Rigidly secure non-movement joints.

Install anchors with separators and isolators to prevent metal corrosion and electrolytic deterioration.

Seal joints watertight unless otherwise indicated.

Metal Protection:

Where aluminum will contact dissimilar metals, protect against galvanic action by painting contact surfaces with primer or applying sealant or tape, or by installing nonconductive spacers as recommended by manufacturer for this purpose.

Where aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous paint.

Install components to drain water passing joints, condensation occurring within framing members, and moisture migrating within the system to exterior.

Set continuous sill members and flashing in full sealant bed as specified in Division 7 Section "Joint Sealants" to produce weather tight installation.

Install components plumb and true in alignment with established lines and grades, and without warp or rack.

Install glazing as specified in Division 8 Section "Glazing."

FIELD QUALITY CONTROL

Testing Agency:  the owner may at his discretion employ a qualified independent testing and inspecting agency to perform field tests and inspections.

Testing Services:  Testing and inspecting of representative areas to determine compliance of installed systems with specified requirements shall take place as follows.  Do not proceed with installation of the next area until test results for previously completed areas show compliance with requirements.

Water Spray Test:  Before installation of interior finishes has begun, areas designated by Architect shall be tested according to AAMA 501.2 and shall not evidence water penetration.

Repair or remove work if test results and inspections indicate that it does not comply with specified requirements.

Additional testing and inspecting on portions of the work that are found to be non compliant will be at Contractor's expense and will be performed to determine compliance of replaced or additional work with specified requirements.

Aluminum-framed assemblies will be considered defective if they do not pass tests and inspections.

Prepare test and inspection reports.

END OF SECTION 08411